## 🔹 1. Block All ICMP (Ping & Other Messages)

To block all ICMP messages (not just ping), use:

access-list 101 deny icmp 192.168.1.10 0.0.0.0 any

access-list 101 permit ip any any

🔹 This blocks all ICMP traffic (including ping, unreachable messages, etc.) from 192.168.1.10.

## 🔹 2. Block a Specific Port (e.g., HTTP - Port 80)

To block HTTP traffic (port 80) from a specific IP:

access-list 101 deny tcp 192.168.1.10 0.0.0.0 any eq 80

access-list 101 permit ip any any

🔹 This prevents 192.168.1.10 from accessing any website using HTTP.

## 🔹 3. Block HTTPS Traffic (Port 443)

To block secure web browsing (HTTPS, port 443):

access-list 101 deny tcp 192.168.1.10 0.0.0.0 any eq 443

access-list 101 permit ip any any

🔹 This prevents secure browsing on HTTPS websites.

## 🔹 4. Block FTP (File Transfer Protocol - Ports 20 & 21)

To prevent file transfers using FTP:

access-list 101 deny tcp 192.168.1.10 0.0.0.0 any eq 21

access-list 101 deny tcp 192.168.1.10 0.0.0.0 any eq 20

access-list 101 permit ip any any

🔹 This blocks both FTP control (port 21) and data transfer (port 20).

## 🔹 5. Block SMTP (Email - Port 25)

To prevent a device from sending emails via SMTP:

access-list 101 deny tcp 192.168.1.10 0.0.0.0 any eq 25

access-list 101 permit ip any any

🔹 This blocks outbound email traffic on SMTP (port 25).

## 🔹 6. Block a Specific Website (Using Domain Name Resolution)

Since ACLs don’t filter by domain names, you need to block the DNS resolution (port 53) first:

access-list 101 deny udp 192.168.1.10 0.0.0.0 any eq 53

access-list 101 permit ip any any

🔹 This prevents 192.168.1.10 from resolving website addresses into IPs, effectively blocking web access.

## 🔹 7. Block a Whole Network (e.g., 192.168.2.0/24)

To block an entire subnet:

access-list 101 deny ip 192.168.2.0 0.0.0.255 any

access-list 101 permit ip any any

🔹 This denies all traffic from 192.168.2.0/24 to any destination.

## 🔹 8. Block SSH (Port 22)

To prevent SSH access from a specific IP:

access-list 101 deny tcp 192.168.1.10 0.0.0.0 any eq 22

access-list 101 permit ip any any

🔹 This prevents 192.168.1.10 from connecting via SSH.

## 🔹 9. Block Telnet (Port 23)

To block Telnet access:

access-list 101 deny tcp 192.168.1.10 0.0.0.0 any eq 23

access-list 101 permit ip any any

🔹 This prevents 192.168.1.10 from connecting via Telnet.

## 🔹 10. Block YouTube (Using IP Address Blocking)

YouTube changes IPs frequently, but you can block Google’s IP ranges:

access-list 101 deny ip 192.168.1.10 0.0.0.0 142.250.0.0 0.0.255.255

access-list 101 permit ip any any

🔹 This blocks access to Google servers that serve YouTube.

## How to Apply an ACL to an Interface?

Once you've created the ACL, apply it to an interface:

interface FastEthernet0/0

ip access-group 101 in

exit

- in → Blocks traffic coming into the router.

- Use out instead if you want to block outgoing traffic.

## How to Verify ACLs?

Check which ACLs are active:

show access-lists 101

or

show running-config | section access-list