Midterm I Re-examination (Solution)

Spring 2023, 20th March 2023, 11:30 am – 12:30 pm

Question 1: 	 	 	 	 	 	 	 	 	CLO#: 01 [10 Points]

Give short answers of the following.

- What is the conductor in fiber optic cable?

Solution: Glass

- Provide any three factors that affect the propagation of wireless signals.

Solution:

reflection

obstruction by objects

Interference/noise

- The same wire in coax cable is used, both for sending and receiving data, so number of downloads and uploads should be equal possible. But in reality, the downstream (download) rate is usually higher than upstream (upload) rate, how?

Solution: Because more channels (frequency bands) are allocated for download, than for upload.

- Which wireless communication technology supports the highest data rate?

Solution: WiFi (WLAN)

- Which physical wired medium supports the highest data rate?

Solution: Fiber Optic Cable.

Question 2:  	 	 	 	           	 	 	 	 	  CLO#: 01 [10 Points]

A user in Karachi, connected to the internet via a 2 Mbps connection retrieves a 25 Kbytes web page from a web server in Islamabad, where the page references 3 images of 200 Kbytes each. Assume that the one way propagation delay is 20 ms.

- How long does it take for the page (including images) to appear on the user’s screen, assuming non persistent HTTP using a single connection at a time?

Solution:

With no persistent HTTP, one TCP connection is established for each page and images. Each object requires 2*RTT plus transmission time.

Each object 2*RTT = 2 * 20ms = 40ms,

Total objects = 4,

So total RTT = 4 * 40ms = 160ms = 0.16

Transmission time = Object Size / data rate

Since, objects are specified in Bytes, while speed in bits, so bytes will be converted into bits, by multiplying with 8.

( (25KB * 8) + (3 * 200KB * 8) ) / 2 Mbps

( 0.2 Mbits + 4.8Mbits ) / 2Mbps

5 Mbits / 2 Mbps

2.5sec

Total time = 2.5 + 0.16 = 2.66sec

- What would become the time for the page (including images) to appear on the user’s screen if he used persistent HTTP without pipelining in place of non-persistent HTTP?

Solution:

Witt persistent HTTP, one RTT for establishing TCP, and one RTT for closing TCP (closing TCP is not required in non-persistent, as it is automatically closed after single object request). In addition, one RTT is required for requesting each object. So total RTTs will be 6, 2 for establishing and terminating TCP connection, and 4 for each object request. Further transmission time for each object will be added to get the total time.

Total time = 6 * RTT + Transmission time

6 * 20msec + 2.5sec

120 msec + 2.5sec

2.62sec

Question 3:  	 	 	 	 	 	 	 	 	CLO#: 02 [10 Points]

You send an e-mail message to your friend amir@gmail.com with a copy to your other friend marco@live.com. The subject of the message is “Pizza” and the text of the message is “Let’s go out for a pizza tonight”.

- Assuming your e-mail address is student@liu.edu.lb and that your mail user-agent is configured to use the mail server liu.edu.lb, write the complete SMTP exchange between your user agent and the mail server of your friends, including the whole body of the message.

Solution:

From: student@liu.edu.lb

To: amir@gmail.com

CC: marco@live.com

Subject: Pizza

Let’s go out for a pizza tonight.

Is it SMTP header and message format, which we actually taught in class?

.

After Subject, we have a different SMTP interaction, in which sender, receiver email addresses are verified by the receiving mail servers.

- Name 2 protocols that your friends can use to access their inbox and read the mail message.

Solution:

IMAP (or POP) and/or HTTP.

Does really SMTP is used to retrieve email???

- What do you have to do in order to send a photo of the pizza attached to your mail message?

Solution:

Does we taught this in class? Even not in our text book, so why we give verily out of course scope questions???

MIME (Multimedia Mail Extension)

Question 4:  	 	 	 	 	 	 	 	 	CLO#: 02 [10 Points]

You want to register your company “Securiteam” website (securiteam.org with IP address “10.0.0.1”) and mail server (mail.securiteam.org with IP address “172.16.5.25”) into the internet.

- What type of DNS server contains the records for the IP addresses of the website and the mail server of your company? Give the two corresponding resource records.

Solution:

(securiteam.org, 10.0.0.1, A)

(mail.securiteam.org, 172.16.5.25, MX)

Oh, can we ever use private IP over the internet???

- If an internet user is searching for the company website’s IP for the first time, what are in order, the DNS servers that the user’s local DNS server need to query?

Solution:

Root -> TLD -> authoritative.

Question 5:  	 	 	 	 	 	 	 	 	 	CLO#: 01 [10 Points]

Figure

1

Consider the scenario in the Figure 1, in which a server is connected to a router by a 100 Mbps link, with a 100 ms propagation delay.  That router in turn is connected to two routers, each over a 25 Mbps link with a 200 ms propagation delay. A Gbps link connects a host and a cache (when present) to each of these routers; this link, being a local area network, has a propagation delay that is essentially zero.  All packets in the network are 10,000 bits long.

- What is the end-to-end delay from when a packet is transmitted by the server to when it is received at a host?  Assume that there are no caches, that there is no queuing delay at a link, and that the node (router) packetprocessing delays are also zero.

Solution:

For packets of size 10,000 bits:

(10Kb / 100,000Kbps) 100 usec will be needed to send the packet over the 100Mbps link(L/R1)

(10Kb / 25,000Kbps) 400 usec will be needed to send over the 25Mbps link (L/R2).

(10Kb / 1,000,000Kbps) 10 usec will be needed to send over the 1Gbps link (L/R3).

Sum of the three-link transmission is 510 usec

Sum of all propagation is 100 + 200 = 300ms

Thus, the total end-to-end delay is 300.51 msec.

- First assume that client hosts send requests for files directly to the server (i.e., the caches are off). What is the maximum rate at which the server can deliver data to a single client, assuming no other clients are making requests?

Solution:

Server can send at the max of the bottleneck link: 25Mbps.

- Again assume that only one client is active, but now suppose the caches are HTTP caches and are turned on.  A client HTTP GET is always first directed to its local cache. 50% of the requests can be satisfied by the local cache. What is the maximum rate at which this client can receive data in this scenario?

Solution:

50% of the requests can be delivered at 25Mbps

50% of the requests can be delivered at 1Gbps.

So the average rate is 25 * 0.5 + 1000 * 50 = 512.5Mpbs.

- Now suppose that the clients in both LANs are active and the HTTP caches are on, as in iii) above. 50% of the requests can be satisfied by the local cache. What is the maximum rate at which each client can receive data, in this scenario?

Solution:

Answer is same as in iii.

25Mbps link remains bottleneck link, which is not shared between LANs.

Note that we assume that the 100Mbps link is shared at fine granularity, so that each client gets up to 50Mbps over that link.

25Mbps can be solution???

- Now suppose the 100 Mbps link is replaced by a 25 Mbps link.  Repeat question iv) above in this new scenario.

Solution:

Note that we assume that the 25Mbps link is shared at fine granularity, so that each client gets up to 12.5Mbps over that link. So, is the bottleneck link.

25Mbps link is not shared between LANs.

Thus,

50% of the requests can be delivered at 12.5Mbps

50% of the requests can be delivered at 1Gbps.

So the average rate is 12.5 * 0.5 + 1000 * 50 = 506.25Mpbs.